1号楼6楼信息科机房网络改造方案

- 网络改造背景

- 优化网络结构

- 现网络拓扑图

- 改造后的网络拓扑图

- 改造前后网络端口对照表：

4.1、电口对照表：H3C S7506E : 华为 S7703

4.2、光口对照表：H3C S7506E : 华为 S7703

- 机柜设备布置图

辅助材料准备清单

| 名称 | 标准、规格 | 数量 |
| --- | --- | --- |
| PDU单元 | 16A，大于10孔 | 2 |
| 光纤跳线 | 单模 |  |
| 网络跳线 | CAT6类（屏蔽线） |  |
| 打印标签 |  |  |

- 配置相关的2台交换机

- 配置新购华为Quidway S7703的交换机（可提前配置）

- 基础配置

sysname HUAWEI

lldp enable

# 配置NTP

clock timezone BJ add 08:00:00

ntp-service server disable

ntp-service ipv6 server disable

ntp-service unicast-server 192.168.89.200

# 配置SSH及用户

sftp server enable

stelnet server enable

ssh user zxyy

ssh user zxyy authentication-type password

ssh user zxyy service-type stelnet

- VLAN配置信息统计表：

| 楼层 | 现有端口 -> 对应华为端口 | VLAN 透传范围 |
| --- | --- | --- |
| 信息科交换机 | G2/0/24 -> GE2/0/24 | 25 31 40 62 300 |
| 急诊1楼 | G2/0/3 -> GE2/0/1 | 24 30 |
| 急诊5楼 | G4/0/20 -> GE3/1/4 | 23 25 101 104 300 to 301 |
| 1号楼C座影像 | G4/0/38 -> GE3/1/6 | 24 31 |
| 3号楼 | G4/0/9 -> GE3/1/0 | 30 to 33 |
| 影像诊断 | G4/0/12 -> GE3/1/1 | 30 to 31 101 300 |
| 门诊1楼 | G4/0/15 -> GE3/1/2 | 26 44 to 46 101 104 |
| 门诊3、2楼 | G4/0/18 -> GE3/1/3 | 41 to 42 101 104 |
| 门诊4、5、6楼 | G4/0/23 -> GE3/1/5 | 42 to 43 104 300 2013 to 2014 |
| 到5号楼外网交换机 | GE4/0/47 -> GE3/1/7 | 100 to 101 104 1000 |
| 到2号楼内网交换机 | 原上行接DELL -> XG3/0/22, 23 | 62 94 119 to 120 150 300 to 301 1000 2013 3001 |
| 外接端口实际 透传VLAN汇总： | { 23, 24, 25, 26, 30, 31, 32, 33, 41, 42, 43, 44, 45, 46, 62, 94, 100, 101, 104, 119, 120, 150, 300, 301, 1000, 2013, 2014, 3001, 3002} 注：迁移到华为交换机后，3002去掉，加上3001（路由，VRV管理口） |  |

vlan batch 23 to 26 30 to 32

vlan batch 40 to 46 62

vlan batch 100 101 104

vlan batch 119 120 162 300 1000 2013

vlan batch 3001

- VLAN 三层接口配置信息：

| VLAN ID | IP地址 | Description <修改描述> | 使用单位 |
| --- | --- | --- | --- |
| 17 | 192.168.17.254 |  | 没用，去掉 |
| 23 | 192.168.23.254 | 1c#-1 | 皮肤病医院：负1楼 |
| 24 | 192.168.24.254 | 1c#1 | 皮肤病医院：1楼 |
| 25 | 192.168.25.254 | 1c#2-5 | 皮肤病医院：2~5楼 |
| 26 | 192.168.26.254 | 1a#5_jyk | 门诊5楼：检验科 |
| 29 | 192.168.29.254 |  | 没用，去掉 |
| 30 | 192.168.30.254 |  | 3号楼-影像楼 |
| 31 | 192.168.31.254 |  | 3号楼-影像楼 |
| 32 | 192.168.32.254 |  | 3号楼-影像楼 |
| 33 | 192.168.33.254 |  | 没用，去掉 |
| 40 | 192.168.40.254 |  | 职工之家 |
| 41 | 192.168.41.254 |  | 门诊1楼 |
| 42 | 192.168.42.254 |  | 门诊2楼 |
| 43 | 192.168.43.254 |  | 门诊3楼 |
| 44 | 192.168.44.254 |  | 门诊4楼 |
| 45 | 192.168.45.254 |  | 门诊5楼 |
| 46 | 192.168.46.254 |  | 门诊6楼 |
| 61 | 192.168.61.254 |  | 没用，去掉 |
| 62 | 192.168.62.254 |  | 信息科 |
| 66 | 192.168.66.254 |  | 没用，去掉 |
| 74 | 192.168.74.254 | kfyy-ip-2 | 没用，去掉 |
| 91 |  | wvlanAP | 没用，去掉 |
| 94 |  |  | 没用，去掉 |
| 100 |  |  | 外网 |
| 101 |  |  | 外网 |
| 104 |  |  | 外网 |
| 119 | 192.168.15.1 | ap-apply | 锐捷内网WiFi，重症 覆盖5号楼、2号楼，用户接入 |
| 120 |  | ap-manage_171.19.100.1 | 锐捷内网WiFi，管理AP |
| 150 |  |  | 没用，去掉 |
| 162 | 192.168.162.254 | anquan-guanli | 安全设备管理， 王凌宇提前调整 |
| 210 | 192.168.110.253 |  | 没用，去掉 |
| 300 | 10.102.3.249 | ycyxzx-sx | 远程会诊-视频 VLAN保留，IP去掉 |
| 301 |  |  | 没用，去掉 |
| 350 | 10.107.3.1/30 |  | 没用，去掉 |
| 551 | 172.100.100.1/28 |  | 没用，去掉 |
| 1000 | 192.168.100.254/24 |  | 部分路由引用 |
| 1098 | 10.98.20.114/28 |  | 没用，去掉 |
| 2013 |  | youjiali-zx | 尤加利，动态心电 |
| 2014 |  | kangtai-zx | 没用，去掉 |
| 2994 | 172.17.200.8/28 | 新郑 | 没用，去掉 |
| 2996 | 172.16.96.1/28 |  | 没用，去掉 |
| 2997 | 172.16.97.1/28 |  | 没用，去掉 |
| 2999 | 172.16.99.2/30 |  | 没用，去掉 |
| 3002 |  |  | 迁移后，去掉 |
| 3010 |  | liantong_vp_BenBu_10.5.1.2 | 没用，去掉 |
| 3012 |  | liantong_VPN_gx_10.5.2.2 | 没用，去掉 |
| 3014 |  | liantong_VPN_kf_10.5.4.2 | 没用，去掉 |
| 3016 |  | liantong_VPN_yx_10.5.6.2 | 没用，去掉 |
| 3018 |  | dianxin_VPN_10.5.8.2 | 没用，去掉 |
| 3019 |  | dianxin_VPN_10.5.9.2 | 没用，去掉 |
| 3020 |  | dianxin_VPN_10.5.10.2 | 没用，去掉 |
| 3071 | 10.5.7.1/30 | dianxin_VPN_mazi_10.5.7.1 | 没用，去掉 |
| 3072 | 10.5.8.1/30 | dianxin_VPN_yx_10.5.8.1 | 没用，去掉 |

VLAN 3层接口：

| 23 | 192.168.23.254 | 1c#-1 | 皮肤病医院：负1楼 |
| --- | --- | --- | --- |
| 24 | 192.168.24.254 | 1c#1 | 皮肤病医院：1楼 |
| 25 | 192.168.25.254 | 1c#2-5 | 皮肤病医院：2~5楼 |
| 26 | 192.168.26.254 | 1a#5_jyk | 门诊5楼：检验科 |
| 30 | 192.168.30.254 |  | 3号楼-影像楼 |
| 31 | 192.168.31.254 |  | 3号楼-影像楼 |
| 32 | 192.168.32.254 |  | 3号楼-影像楼 |
| 40 | 192.168.40.254 |  | 职工之家 |
| 41 | 192.168.41.254 |  | 门诊1楼 |
| 42 | 192.168.42.254 |  | 门诊2楼 |
| 43 | 192.168.43.254 |  | 门诊3楼 |
| 44 | 192.168.44.254 |  | 门诊4楼 |
| 45 | 192.168.45.254 |  | 门诊5楼 |
| 46 | 192.168.46.254 |  | 门诊6楼 |
| 62 | 192.168.62.254 |  | 信息科 |
| 119 | 192.168.15.1 | ap-apply | 锐捷内网WiFi，重症 覆盖5号楼、2号楼，用户接入 |
| 162 | 192.168.162.254 | anquan-guanli | 安全设备管理， 王凌宇提前调整 |
| 300 | 10.102.3.249 | ycyxzx-sx | 远程会诊-视频测试，IP已迁移 |
| 1000 | 192.168.100.254/24 |  | 部分路由引用 |
| 3001 | 172.16.101.2/24 | 与2号楼互联互通 | 与2号楼核心互联 |

| interface Vlan-interface23 description 1c#-1 ip address 192.168.23.254 255.255.255.0 # interface Vlan-interface24 description jz1lou ip address 192.168.24.254 255.255.255.0 # interface Vlan-interface25 description pfk ip address 192.168.25.254 255.255.255.0 # interface Vlan-interface26 description jyk ip address 192.168.26.254 255.255.255.0 | interface Vlan-interface23 description 1c#-1 ip address 192.168.23.254 255.255.255.0 # interface Vlan-interface24 description jz1lou ip address 192.168.24.254 255.255.255.0 # interface Vlan-interface25 description pfk ip address 192.168.25.254 255.255.255.0 # interface Vlan-interface26 description jyk ip address 192.168.26.254 255.255.255.0 |
| --- | --- |
| interface Vlan-interface30 ip address 192.168.30.254 255.255.255.0 # interface Vlan-interface31 ip address 192.168.31.254 255.255.255.0 # interface Vlan-interface32 ip address 192.168.32.254 255.255.255.0 | interface Vlan-interface30 ip address 192.168.30.254 255.255.255.0 # interface Vlan-interface31 ip address 192.168.31.254 255.255.255.0 # interface Vlan-interface32 ip address 192.168.32.254 255.255.255.0 |
| interface Vlan-interface40 ip address 192.168.40.254 255.255.255.0 # interface Vlan-interface41 ip address 192.168.41.254 255.255.255.0 # interface Vlan-interface42 ip address 192.168.42.254 255.255.255.0 # interface Vlan-interface43 ip address 192.168.43.254 255.255.255.0 interface Vlan-interface44 ip address 192.168.44.254 255.255.255.0 # interface Vlan-interface45 ip address 192.168.45.254 255.255.255.0 # interface Vlan-interface46 description mz-6lou ip address 192.168.46.254 255.255.255.0 | interface Vlan-interface40 ip address 192.168.40.254 255.255.255.0 # interface Vlan-interface41 ip address 192.168.41.254 255.255.255.0 # interface Vlan-interface42 ip address 192.168.42.254 255.255.255.0 # interface Vlan-interface43 ip address 192.168.43.254 255.255.255.0 interface Vlan-interface44 ip address 192.168.44.254 255.255.255.0 # interface Vlan-interface45 ip address 192.168.45.254 255.255.255.0 # interface Vlan-interface46 description mz-6lou ip address 192.168.46.254 255.255.255.0 |
| interface Vlan-interface62 description xxk ip address 192.168.62.254 255.255.255.0 | interface Vlan-interface62 description xxk ip address 192.168.62.254 255.255.255.0 迁移到华为防火墙 |
| interface Vlan-interface119 description ap-apply ip address 192.168.15.1 255.255.255.0 | interface Vlan-interface119 description ap-apply ip address 192.168.15.1 255.255.255.0 |
| interface Vlan-interface162 description anquan-guanli ip address 192.168.162.254 255.255.255.0 | interface Vlan-interface162 description anquan-guanli ip address 192.168.162.254 255.255.255.0 网关需要迁移到5号楼DELL交换机上，配置相关静态路由表 |
| interface Vlan-interface300 description ycyxzx-sx ip address 10.102.3.249 255.255.255.0 | interface Vlan-interface300 description ycyxzx-sx ip address 10.102.3.249 255.255.255.0 IP地址已经迁移到2号楼核心交换机12708 |
| interface Vlan-interface1000 ip address 192.168.100.1 255.255.255.0 vrrp vrid 1 virtual-ip 192.168.100.254 vrrp vrid 1 priority 220 packet-filter 3003 inbound packet-filter 3003 outbound | interface Vlan-interface1000 ip address 192.168.100.254 255.255.255.0 需要迁移到2号楼核心交换机12708，配置相关静态路由表 |
|  | interface Vlan-interface3001 description TO_HW_S12708_G7/0/2-3 ip address 172.16.101.2 255.255.255.240 |

- 配置从H3C S7506E迁移过来的电口

| 原H3C S7506E交换机 | 新购华为 S7703交换机 |
| --- | --- |
| interface GigabitEthernet2/0/2 port link-mode bridge description neiwang-wlan port link-type trunk undo port trunk permit vlan 1 port trunk permit vlan 119 to 120 loopback-detection enable packet-filter 3000 inbound packet-filter 3000 outbound qos apply policy refuse-445 inbound qos apply policy refuse-445 outbound dhcp-snooping trust | interface GigabitEthernet2/0/0 port link-mode bridge description neiwang-wlan port link-type trunk undo port trunk permit vlan 1 port trunk permit vlan 119 to 120 loopback-detection enable |
| interface GigabitEthernet2/0/3 port link-mode bridge description 192.168.24.249_47port port link-type trunk undo port trunk permit vlan 1 port trunk permit vlan 24 30 mirroring-group 2 mirroring-port both | interface GigabitEthernet2/0/1 port link-mode bridge description 192.168.24.249_47port port link-type trunk undo port trunk permit vlan 1 port trunk permit vlan 24 30 |
| interface GigabitEthernet2/0/12 port link-mode bridge description sangfor_tanzhen mirroring-group 3 monitor-port | interface GigabitEthernet2/0/2 port link-mode bridge description sangfor_tanzhen 镜像口---mirror |
| interface GigabitEthernet2/0/14 port link-mode bridge description tanzheng-manage_port_192.168.100.198 port access vlan 1000 | interface GigabitEthernet2/0/3 port link-mode bridge description tanzheng-manage_port_192.168.100.198 port access vlan 1000 深信服探针 |
| interface GigabitEthernet2/0/15 port link-mode bridge description TO_VRV_zhunru_172.16.101.6_mirro_port mirroring-group 2 monitor-port | interface GigabitEthernet2/0/4 port link-mode bridge description TO_VRV_zhunru_172.16.101.6_mirro_port |
| interface GigabitEthernet2/0/7 port link-mode route packet-filter 3000 inbound packet-filter 3000 outbound ip address 172.30.200.1 255.255.255.252 | interface GigabitEthernet2/0/5 desc XinZhengYuanQu port link-mode route ip address 172.30.200.1 255.255.255.252 |
| interface GigabitEthernet2/0/24 port link-mode bridge description xxk-jieru_192.168.62.252 port link-type trunk undo port trunk permit vlan 1 port trunk permit vlan 25 31 40 62 300 packet-filter 3003 inbound packet-filter 3003 outbound mirroring-group 2 mirroring-port both | interface GigabitEthernet2/0/24 port link-mode bridge description xxk-jieru_192.168.62.2 52 port link-type trunk undo port trunk permit vlan 1 port trunk permit vlan 25 31 40 62 300 |
| 北信源准入管理口 DELL交换机 Te1/5 | interface GigabitEthernet2/0/6 description beixinyuan-zhunru port link-type access port default vlan 3001 |

- 配置从H3C S7506E迁移过来的光口

| 原H3C S7506E交换机 | 新购华为 S7703交换机 |
| --- | --- |
| interface GigabitEthernet4/0/9 port link-mode bridge description to 3# port link-type trunk undo port trunk permit vlan 1 port trunk permit vlan 30 to 33 qos apply policy refuse-445 inbound qos apply policy refuse-445 outbound mirroring-group 2 mirroring-port both | interface GigabitEthernet3/1/0 port link-mode bridge description to 3# port link-type trunk undo port trunk permit vlan 1 port trunk permit vlan 30 to 33 |
| interface GigabitEthernet4/0/12 port link-mode bridge description to yixizhenduan port link-type trunk undo port trunk permit vlan 1 port trunk permit vlan 30 to 31 101 300 qos apply policy refuse-445 inbound qos apply policy refuse-445 outbound mirroring-group 2 mirroring-port both | interface GigabitEthernet3/1/1 port link-mode bridge description to yixizhenduan port link-type trunk undo port trunk permit vlan 1 port trunk permit vlan 30 to 31 101 300 |
| interface GigabitEthernet4/0/15 port link-mode bridge description mz4lou port link-type trunk undo port trunk permit vlan 1 port trunk permit vlan 26 44 to 46 101 104 qos apply policy refuse-445 inbound qos apply policy refuse-445 outbound mirroring-group 2 mirroring-port both | interface GigabitEthernet3/1/2 port link-mode bridge description mz4lou port link-type trunk undo port trunk permit vlan 1 port trunk permit vlan 26 44 to 46 101 104 |
| interface GigabitEthernet4/0/18 port link-mode bridge description mz1lou port link-type trunk undo port trunk permit vlan 1 port trunk permit vlan 41 to 42 101 104 qos apply policy refuse-445 inbound qos apply policy refuse-445 outbound mirroring-group 2 mirroring-port both | interface GigabitEthernet3/1/3 port link-mode bridge description mz1lou port link-type trunk undo port trunk permit vlan 1 port trunk permit vlan 41 to 42 101 104 |
| interface GigabitEthernet4/0/20 port link-mode bridge description to jz5# port link-type trunk undo port trunk permit vlan 1 port trunk permit vlan 23 25 101 104 300 to 301 qos apply policy refuse-445 inbound qos apply policy refuse-445 outbound mirroring-group 2 mirroring-port both | interface GigabitEthernet3/1/4 port link-mode bridge description to jz5# port link-type trunk undo port trunk permit vlan 1 port trunk permit vlan 23 25 101 104 300 |
| interface GigabitEthernet4/0/23 port link-mode bridge description mz-3luo port link-type trunk undo port trunk permit vlan 1 port trunk permit vlan 42 to 43 104 300 2013 to 2014 qos apply policy refuse-445 inbound qos apply policy refuse-445 outbound mirroring-group 2 mirroring-port both | interface GigabitEthernet3/1/5 port link-mode bridge description mz-3luo port link-type trunk undo port trunk permit vlan 1 port trunk permit vlan 42 to 43 101 104 300 2013 |
| interface GigabitEthernet4/0/38 port link-mode bridge description yingxiang_192.168.24.250 port link-type trunk undo port trunk permit vlan 1 port trunk permit vlan 24 31 mirroring-group 2 mirroring-port both | interface GigabitEthernet3/1/6 port link-mode bridge description yingxiang_192.168.24.250 port link-type trunk undo port trunk permit vlan 1 port trunk permit vlan 24 31 |
| interface GigabitEthernet4/0/47 port link-mode bridge description TO_5haolou_H3C_S7003E_GE0/0/19 port link-type trunk port trunk permit vlan 1 70 100 to 101 104 1000 stp disable mirroring-group 2 mirroring-port both # 改由自己接入1号楼外网汇聚交换机。 # vlan 1000去掉后：影响62段上网、外网上OA、外网堡垒机、省工伤医保、医联体VPN。。。 | interface GigabitEthernet2/0/23 description TO_Huawei_S5720(wai)_192.168.101.241_p23 port link-type trunk undo port trunk allow-pass vlan 1 port trunk allow-pass vlan 100 to 101 104 stp disable 连接1号楼外网汇聚交换机 192.168.101.241 # 同时配置好内外网防火墙 |

- 配置与2号楼核心互联接口

使用：XG3/0/22、XG3/0/23 接口连接2号楼核心交换机

interface Eth-Trunk8 description TO_2haolou_HW_S12708 port link-type trunk undo port trunk allow-pass vlan 1 port trunk allow-pass vlan 62 119 to 120 300 1000 2013 3001 mode lacp load-balance dst-mac

interface XGigabitEthernet3/0/22 description TO_2haolou_HW_S12708_XG7/0/2 eth-trunk 8

interface XGigabitEthernet3/0/23 description TO_2haolou_HW_S12708_XG7/0/3 eth-trunk 8

- 配置端口镜像组到深信服探针、北信源准入系统

已配置 observe-port 2 interface GigabitEthernet2/0/4 observe-port 3 interface GigabitEthernet2/0/2

- 配置生成树协议

生成树保护 stp bpdu-protection

- 192.168.62.0段，IP-MAC地址绑定

arp static 192.168.62.247 0001-0001-0001 arp static 192.168.62.250 0001-0001-0001 arp static 192.168.62.175 ace0-103b-7f4f arp static 192.168.62.158 9ceb-e85f-18f4 arp static 192.168.62.204 0001-0002-0004 arp static 192.168.62.219 0001-0008-0009 arp static 192.168.25.73 0001-0002-0008 arp static 192.168.62.218 0001-0002-0003 arp static 192.168.25.234 0001-0002-0003 arp static 192.168.62.154 0001-0030-0200 arp static 192.168.62.178 0001-0030-0200 arp static 192.168.62.184 0001-0030-0200 arp static 192.168.62.186 0001-0030-0200 arp static 192.168.62.188 0001-0030-0200 arp static 192.168.62.193 0001-0030-0200 arp static 192.168.62.194 0001-0030-0200 arp static 192.168.62.198 0001-0030-0200 arp static 192.168.62.199 0001-0030-0200 arp static 192.168.62.202 0001-0030-0200 arp static 192.168.62.203 0001-0030-0200 arp static 192.168.62.205 0001-0030-0200 arp static 192.168.62.206 0001-0030-0200 arp static 192.168.62.207 0001-0030-0200 arp static 192.168.62.208 0001-0030-0200 arp static 192.168.62.209 0001-0030-0200 arp static 192.168.62.211 0001-0030-0200 arp static 192.168.62.212 0002-0021-0010 arp static 192.168.62.213 0001-0030-0200 arp static 192.168.62.214 0001-0030-0200 arp static 192.168.62.215 0001-0030-0200 arp static 192.168.62.217 0001-0030-0200 arp static 192.168.62.221 0001-0030-0200 arp static 192.168.62.223 0001-0030-0200 arp static 192.168.62.224 0001-0030-0200 arp static 192.168.62.225 0001-0030-0200 arp static 192.168.62.226 0001-0030-0200 arp static 192.168.62.227 0001-0030-0200 arp static 192.168.62.229 0001-0030-0200 arp static 192.168.62.231 0001-0030-0200 arp static 192.168.62.243 0001-0030-0200 arp static 192.168.62.245 0001-0030-0200 arp static 192.168.62.246 0001-0030-0200 arp static 192.168.62.251 0001-0030-0200 arp static 192.168.62.242 0010-0020-0033 arp static 192.168.62.29 0010-0020-0033 arp static 192.168.62.39 0010-0020-0033 arp static 192.168.62.78 0010-0020-0033 arp static 192.168.62.91 0010-0020-0033 arp static 192.168.62.92 0010-0020-0033 arp static 192.168.62.93 0010-0020-0033 arp static 192.168.62.94 0010-0020-0033 arp static 192.168.62.96 0010-0020-0033 arp static 192.168.25.230 0000-1111-2222 arp static 192.168.25.231 0001-0003-0005 arp static 192.168.25.232 0002-0005-0006 arp static 192.168.62.222 0001-1234-3215 arp static 192.168.62.128 d43a-6507-19a6 arp static 192.168.62.143 9c5a-4486-7825 arp static 192.168.62.113 507b-9df7-8529 arp static 192.168.62.23 8c16-4555-61ec arp static 192.168.62.14 f80f-4170-dbee arp static 192.168.62.7 3c2c-30e7-b0a2 arp static 192.168.62.17 80fa-5b3d-e557 arp static 192.168.62.239 6c4b-904b-5a27 arp static 192.168.62.195 28d2-44ba-d170 arp static 192.168.62.70 9ceb-e85f-18f4 arp static 192.168.62.168 54e1-ad2f-5ebe arp static 192.168.62.164 4c34-881a-086f arp static 192.168.62.174 507b-9d29-9431 arp static 192.168.62.12 0001-0003-0019 arp static 192.168.62.155 0001-0003-0500 arp static 192.168.62.180 0001-0003-0500 arp static 192.168.62.228 0001-0003-0500 arp static 192.168.62.230 0001-0003-0500 arp static 192.168.62.233 0001-0003-0500 arp static 192.168.62.234 0001-0003-0501 arp static 192.168.62.240 0001-0003-0501 arp static 192.168.62.241 0001-0003-0501 arp static 192.168.62.244 0001-0003-0501 arp static 192.168.62.253 0001-0003-0501 arp static 192.168.62.62 0001-0003-0501 arp static 192.168.62.179 0001-0003-0005 arp static 192.168.62.25 0001-0003-0005 arp static 192.168.62.26 0001-0003-0005 arp static 192.168.62.27 0001-0003-0005 arp static 192.168.62.24 0001-0003-0005 arp static 192.168.62.249 e4f0-04e1-a41a arp static 192.168.62.55 c809-a808-f78c arp static 192.168.62.120 d481-d7f9-08ed arp static 192.168.62.127 000e-c68f-af55 arp static 192.168.62.237 a44c-c879-a13a arp static 192.168.62.60 000e-c6b9-ba01 arp static 192.168.62.165 9c5a-4489-4297 arp static 192.168.62.30 54ee-75cf-9f9f arp static 192.168.62.107 002b-678f-3987 arp static 192.168.62.110 002b-678f-3987 arp static 192.168.62.34 88d7-f63a-a0ee arp static 192.168.62.133 c8d3-ffdd-d0ec arp static 192.168.62.54 80fa-5b0c-8e6b arp static 192.168.62.123 308d-99aa-1ec2 arp static 192.168.62.35 28d2-44bb-5161 arp static 192.168.62.161 002b-678f-3987 arp static 192.168.62.171 00e0-4c72-8b93 arp static 192.168.62.126 f076-1c14-bd80 arp static 192.168.62.129 e4b9-7a5d-8c79 arp static 192.168.62.103 acfd-ce9b-f6e5 arp static 192.168.62.16 0001-0002-0003 arp static 192.168.62.132 507b-9d0e-8ce3 arp static 192.168.62.31 0001-0002-0006 arp static 192.168.62.71 f80f-41a6-f966 arp static 192.168.62.73 000e-c676-23a7 arp static 192.168.62.192 68f7-2825-62c2 arp static 192.168.62.104 bce9-2fb1-60b6 arp static 192.168.62.58 0021-ccd4-2931 arp static 192.168.62.53 5065-f349-ce80 arp static 192.168.62.141 e86a-6453-1d93 arp static 192.168.62.151 98fa-9be0-20b0 arp static 192.168.62.119 b025-aa30-7232 arp static 192.168.62.139 507b-9d22-53e0 arp static 192.168.62.153 100d-0e75-2c72 arp static 192.168.62.105 00e0-4c68-520c arp static 192.168.62.57 c85b-76c2-3b9f arp static 192.168.62.115 507b-9d79-b2a1 arp static 192.168.62.114 0024-3224-4b5e arp static 192.168.62.48 002b-67cf-9069 arp static 192.168.62.3 ccd3-9d9c-2c45 arp static 192.168.62.109 80fa-5b51-c3c0 arp static 192.168.62.5 0010-f370-f1ce arp static 192.168.62.75 f48e-38f1-9e62 arp static 192.168.62.137 54e1-ad55-41b1 arp static 192.168.62.121 d05f-6436-a4ee arp static 192.168.62.116 0021-ccce-f096 arp static 192.168.62.2 0016-31df-d033 arp static 192.168.62.182 8c16-45ea-6974 arp static 192.168.62.170 54ee-75d9-916d arp static 192.168.62.43 1c1b-0d58-4ff4 arp static 192.168.62.32 507b-9d1c-e78f arp static 192.168.62.11 0cc4-7a79-8f71 arp static 192.168.62.144 000e-c6d7-ee61 arp static 192.168.62.88 0023-24e2-6d76 arp static 192.168.62.77 2c16-dba5-e742 arp static 192.168.62.134 00e0-4d71-bdc2 arp static 192.168.62.74 80fa-5b40-d0b1 arp static 192.168.62.13 e45f-0107-e219 arp static 192.168.62.33 b025-aa30-7232 arp static 192.168.62.38 00e0-4c7a-f150 arp static 192.168.62.156 80fa-5b07-9e08 arp static 192.168.62.152 0007-3e98-cbc5 arp static 192.168.62.177 f076-1c14-bd80 arp static 192.168.62.145 c85b-7627-d100 arp static 192.168.62.98 c8f7-5005-a510 arp static 192.168.62.172 507b-9d78-3337 arp static 192.168.62.6 94c6-918e-9ed1 arp static 192.168.62.232 68f7-2810-b8b8 arp static 192.168.62.51 98fa-9bc0-42ba arp static 192.168.62.40 98fa-9bbf-564f arp static 192.168.62.49 f48e-38ee-bb35 arp static 192.168.62.140 000e-c663-8c15 arp static 192.168.62.248 6891-d0d7-5584 arp static 192.168.62.101 e86a-64c7-ea5b arp static 192.168.62.187 8c16-458a-31b8 arp static 192.168.62.159 107d-1a36-8227 arp static 192.168.62.36 98fa-9bc0-4662 arp static 192.168.62.173 68f7-2810-b8b8 arp static 192.168.62.176 507b-9d0e-8ce3 arp static 192.168.62.52 68f7-2811-8a83 arp static 192.168.62.125 1c39-47bc-384a arp static 192.168.62.61 e4b9-7a5d-58aa arp static 192.168.62.150 54ee-759a-87aa arp static 192.168.62.20 484d-7ec8-e53d arp static 192.168.62.21 f8a9-6335-0a77 arp static 192.168.62.142 94c6-918e-9eb4 arp static 192.168.62.149 00e0-4c36-01da arp static 192.168.62.163 2c16-dba1-a5f1 arp static 192.168.62.8 1065-3064-9064 arp static 192.168.62.135 000e-c679-18c3 arp static 192.168.62.124 000e-c662-fccd arp static 192.168.62.28 98fa-9bc0-42ba arp static 192.168.62.15 80fa-5b66-5665 arp static 192.168.62.167 8c16-4568-a363 arp static 192.168.62.46 68f7-28b1-8a8b arp static 192.168.62.147 c018-5016-4e0c arp static 192.168.62.216 c018-5004-5c48 arp static 192.168.62.72 000e-c665-8082 arp static 192.168.62.45 54e1-ad2d-5273 arp static 192.168.62.47 54ee-75ea-38b5 arp static 192.168.62.44 d481-d78e-f2cc arp static 192.168.62.42 4ccc-6a39-26b9 arp static 192.168.62.235 6431-507d-1b0e arp static 192.168.62.166 6c4b-904a-1399 arp static 192.168.62.191 9c5a-4487-10db arp static 192.168.62.183 1c39-4720-1398 arp static 192.168.62.90 9c5a-4487-0445 arp static 192.168.62.4 9829-a65d-6965 arp static 192.168.62.10 6891-d065-f217 arp static 192.168.62.87 1c39-4720-69a3 arp static 192.168.62.111 d45d-6439-d8cf arp static 192.168.62.22 9c5a-4488-0713 arp static 192.168.62.85 b42e-99d7-94e1 arp static 192.168.62.41 1c39-4720-0b75 arp static 192.168.62.102 fc45-9627-cda9 arp static 192.168.62.82 1831-bfbc-7661 arp static 192.168.62.112 9c5a-4428-e652 arp static 192.168.62.67 9c5a-4428-6856 arp static 192.168.62.86 6c4b-904b-59fb arp static 192.168.62.89 9c5a-4428-e755 arp static 192.168.62.84 9c5a-4487-0f6e arp static 192.168.62.80 e86a-6453-1f64 arp static 192.168.62.162 305a-3a0e-7dff arp static 192.168.62.65 4ccc-6a19-34d1 arp static 192.168.62.66 309c-2353-88f6 arp static 192.168.62.196 9c5a-4487-02f0 arp static 192.168.62.157 9c5a-4428-0dad arp static 192.168.62.18 2c60-0c9c-cec1 arp static 192.168.62.106 9c5a-4489-4297 arp static 192.168.62.64 28d2-4484-032f arp static 192.168.62.189 fc45-962a-a8ba arp static 192.168.62.169 9c5a-4487-0f8c arp static 192.168.62.56 0897-98fc-d3fd arp static 192.168.62.108 0897-98fe-cd51 arp static 192.168.62.9 0090-0b42-a6eb arp static 192.168.62.197 5065-f35c-5f13 arp static 192.168.62.79 d8c4-97bf-cdbb arp static 192.168.62.252 2065-8ee0-3fb3 arp static 192.168.62.81 9c5a-4428-e3d1 arp static 192.168.62.181 0897-98fd-72d5 arp static 192.168.62.83 1c39-4720-68ad arp static 192.168.62.69 1c39-4720-6990 arp static 192.168.62.201 5405-db7a-6c32 arp static 192.168.62.122 3456-7548-6781 arp static 192.168.62.68 f02f-74a2-0d3d arp static 192.168.62.50 b07b-2573-d6a2 arp static 192.168.62.63 e86a-64d1-663f arp static 192.168.62.76 00e0-4f71-53b1 arp static 192.168.62.117 d43a-6506-99c6 arp static 192.168.62.100 e86a-6443-b848 arp static 192.168.62.236 a44c-c879-a13a arp static 192.168.62.190 9c5a-4486-7ad5 arp static 192.168.62.136 00e0-4c68-1a70 arp static 192.168.62.220 b4b5-2f82-a79b arp static 192.168.62.185 708b-cd0c-3ec3 arp static 192.168.62.19 00e0-4c68-5e74 arp static 192.168.62.118 8c16-45bc-c623 arp static 192.168.62.210 54ee-75dc-7eeb arp static 192.168.62.238 54e1-ad22-d531 arp static 192.168.62.95 6c02-e094-b030 arp static 192.168.62.160 5405-db7a-646e arp static 192.168.62.138 2c16-dba3-a60d arp static 192.168.62.200 000c-298b-022d arp static 192.168.62.131 54ee-75d9-916d arp static 192.168.62.146 4ccc-6ae0-822f arp static 192.168.62.99 8c8c-aae2-2958 arp static 192.168.62.130 1c69-7aa5-e31c arp static 192.168.62.97 c85b-76a9-e37a arp static 192.168.62.148 68f7-28fb-d886 arp static 192.168.62.59 28d2-4460-ee20 arp static 192.168.62.1 000b-ab80-52a9 62 GigabitEthernet2/0/24

- 合并静态路由表

A）分析DELL S4048T-ON交换机静态路由表

| 现有静态路由 | 对应改造后所需的静态路由说明 |
| --- | --- |
| ip route 0.0.0.0/0 172.16.102.2 | 主要用于192.168.62.0段访问互联网，以及医院办公外网10.10.0.0/16段通过天融信防火墙直接访问内网OA（10.20.1.180）； 本次改造在信息科加配防火墙后，默认路由不在需要。 |
| ip route 10.5.18.0/24 10.5.17.1 | 无效路由 |
| ip route 10.20.0.0/24 172.16.101.1 ip route 10.20.1.0/24 172.16.101.1 ip route 10.20.2.0/24 172.16.101.1 ip route 10.30.0.0/16 172.16.101.1 | # 现服务器网段、以及预留网段： ip route-static 10.20.0.0 24 172.16.101.1 ip route-static 10.20.1.0 24 172.16.101.1 ip route-static 10.20.2.0 24 172.16.101.1 # 下面路由可去掉 # ip route-static 10.30.0.0 16 172.16.101.1 |
| ip route 192.168.1.0/24 172.16.101.1 ip route 192.168.2.0/24 172.16.101.1 ip route 192.168.3.0/24 172.16.101.1 ip route 192.168.4.0/24 172.16.101.1 ip route 192.168.5.0/24 172.16.101.1 ip route 192.168.6.0/24 172.16.101.1 ip route 192.168.7.0/24 172.16.101.1 ip route 192.168.8.0/24 172.16.101.1 ip route 192.168.9.0/24 172.16.101.1 ip route 192.168.10.0/24 172.16.101.1 ip route 192.168.11.0/24 172.16.101.1 ip route 192.168.12.0/24 172.16.101.1 ip route 192.168.14.0/24 172.16.101.1 ip route 192.168.18.0/24 172.16.101.1 ip route 192.168.19.0/24 172.16.101.1 ip route 192.168.20.0/24 172.16.101.1 ip route 192.168.27.0/24 172.16.101.1 ip route 192.168.60.0/24 172.16.101.1 ip route 192.168.63.0/24 172.16.101.1 | # 5号楼网络路由 ip route-static 192.168.1.0 24 172.16.101.1 ip route-static 192.168.2.0 24 172.16.101.1 ip route-static 192.168.3.0 24 172.16.101.1 ip route-static 192.168.4.0 24 172.16.101.1 ip route-static 192.168.5.0 24 172.16.101.1 ip route-static 192.168.6.0 24 172.16.101.1 ip route-static 192.168.7.0 24 172.16.101.1 ip route-static 192.168.8.0 24 172.16.101.1 ip route-static 192.168.9.0 24 172.16.101.1 ip route-static 192.168.10.0 24 172.16.101.1 ip route-static 192.168.11.0 24 172.16.101.1 ip route-static 192.168.12.0 24 172.16.101.1 ip route-static 192.168.14.0 24 172.16.101.1 ip route-static 192.168.18.0 24 172.16.101.1 ip route-static 192.168.19.0 24 172.16.101.1 # ip route-static 192.168.20.0 24 172.16.101.1 ip route-static 192.168.27.0 24 172.16.101.1 ip route-static 192.168.60.0 24 172.16.101.1 ip route-static 192.168.63.0 24 172.16.101.1 |

| ip route 192.168.21.0/24 172.16.101.1 ip route 192.168.22.0/24 172.16.101.1 ip route 192.168.80.0/21 172.16.101.1 ip route 192.168.88.0/24 172.16.101.1 ip route 192.168.89.0/24 172.16.101.1 | # 2号楼网段（内网） ip route-static 192.168.21.0 24 172.16.101.1 ip route-static 192.168.22.0 24 172.16.101.1 ip route-static 192.168.80.0 21 172.16.101.1 ip route-static 192.168.88.0 24 172.16.101.1 ip route-static 192.168.89.0 24 172.16.101.1 |
| --- | --- |
| ip route 192.168.48.0/21 172.16.101.1 ip route 192.168.56.0/22 172.16.101.1 | # 高新院区 ip route-static 192.168.48.0 21 172.16.101.1 ip route-static 192.168.56.0 22 172.16.101.1 |
| ip route 192.168.70.0/24 172.16.101.1 ip route 192.168.75.0/24 172.16.101.1 ip route 192.168.76.0/24 172.16.101.1 ip route 192.168.94.0/24 172.16.101.1 ip route 192.168.95.0/24 172.16.101.1 ip route 192.168.97.0/24 172.16.101.1 ip route 192.168.98.0/24 172.16.101.1 | # 各分院 ip route-static 192.168.70.0 24 172.16.101.1 ip route-static 192.168.75.0 24 172.16.101.1 ip route-static 192.168.76.0 24 172.16.101.1 ip route-static 192.168.94.0 24 172.16.101.1 ip route-static 192.168.95.0 24 172.16.101.1 ip route-static 192.168.97.0 24 172.16.101.1 ip route-static 192.168.98.0 24 172.16.101.1 # 马寨192.168.70.0 # 康复医院192.168.75.0 # 豫欣医院192.168.76.0 # 沟赵卫生院192.168.94.0 # 文化宫路院区192.168.95.0 # 老干部活动中心192.168.97.0 # 五棉社区服务站192.168.97.0 # 纪检委192.168.98.0 # 人大192.168.98.0 |
| ip route 10.40.124.0/24 172.16.101.1 | # 2号楼核心（有部分设备在用，确认？） ip route-static 10.40.124.0 24 172.16.101.1 |
| ip route 59.207.34.0/24 172.16.101.1 ip route 59.207.51.0/24 172.16.101.1 ip route 59.207.61.0/24 172.16.101.1 ip route 10.167.4.0/24 172.16.101.1 ip route 10.224.0.0/16 172.16.101.1 ip route 10.232.0.0/16 172.16.101.1 | # 郑州市卫健委（电子政务外网） ip route-static 59.207.34.0 24 172.16.101.1 ip route-static 59.207.51.0 24 172.16.101.1 ip route-static 59.207.61.0 24 172.16.101.1 # 电子政务外网，到2号楼--> 5号楼DELL -> 10.167.4.2（防火墙USG6309E） ip route-static 10.167.4.0 24 172.16.101.1 # 10.167.4.2 卫健委视频专线防火墙 ip route-static 10.224.0.0 16 172.16.101.1 ip route-static 10.232.0.0 16 172.16.101.1 # 卫生专网，到2号楼--> 5号楼DELL（4.3） -> 10.167.4.2（华为） |

| ip route 172.16.100.0/24 172.16.101.1 ip route 172.16.101.0/24 172.16.101.1 ip route 172.16.103.0/24 172.16.101.1 ip route 11.0.2.0/24 172.16.101.1 | # 三层路由互联地址： ip route-static 172.16.100.0 24 172.16.101.1 ip route-static 172.16.101.0 24 172.16.101.1 ip route-static 172.16.103.0 24 172.16.101.1 ip route-static 11.0.2.0 24 172.16.101.1 |
| --- | --- |
| ip route 10.98.0.0/16 172.16.101.1 | #ip route-static 10.98.0.0 16 172.16.101.1（弃用） |
| ip route 10.100.0.0/16 172.16.101.1 ip route 10.102.0.0/16 172.16.101.1 | ip route-static 10.100.0.0 16 172.16.101.1 ip route-static 10.102.0.0 16 172.16.101.1 # 视频会议（郑大一附院，远程医疗） |
| ip route 15.0.160.35/32 172.16.101.1 | ip route-static 15.0.160.35 32 172.16.101.1 # 柯利尔（建行）到2号楼--> 5号楼DELL（4.3） -> 10.167.4.2（华为） |
| ip route 180.9.0.0/16 172.16.101.1 | ip route-static 180.9.0.0 16 172.16.101.1 # 120（院前急救），H3C SecPath F1000-AK125，通过2号楼，192.168.80.1 admin zxyy@123456 |

B）分析H3C S7506E交换机静态路由表

ip route-static 0.0.0.0 0.0.0.0 192.168.100.253

医联体VPN，以下全部弃用（跟王凌宇确认过） ip route-static 10.2.1.0 255.255.255.0 192.168.199.90 description yilianti_vpn ip route-static 10.2.2.0 255.255.255.0 192.168.75.251 description gaozhaotijian

ROS，以下全部弃用（跟王凌宇确认过） ip route-static 10.5.1.0 255.255.255.0 192.168.100.190 ip route-static 10.5.2.0 255.255.255.0 192.168.100.190 ip route-static 10.5.4.0 255.255.255.0 192.168.100.190 ip route-static 10.5.6.0 255.255.255.0 192.168.100.190 ip route-static 10.5.7.0 255.255.255.0 192.168.100.189 ip route-static 10.5.8.0 255.255.255.0 192.168.100.189 ip route-static 10.5.9.0 255.255.255.0 192.168.100.189 ip route-static 10.5.11.0 255.255.255.0 192.168.100.189 ip route-static 10.5.12.0 255.255.255.0 192.168.100.189 ip route-static 10.5.14.0 255.255.255.0 192.168.100.189 ip route-static 10.5.15.0 255.255.255.0 192.168.100.189 ip route-static 10.5.16.0 255.255.255.0 192.168.100.189 ip route-static 10.5.17.0 255.255.255.0 192.168.100.187 ip route-static 10.5.18.0 255.255.255.0 192.168.100.187 ip route-static 10.5.19.0 255.255.255.0 192.168.100.190 ip route-static 10.5.21.0 255.255.255.0 192.168.100.190

# 医院内网电脑IP段，指向DELL交换机 ## 5号楼网络： ip route-static 192.168.1.0 255.255.255.0 172.16.102.1 ip route-static 192.168.2.0 255.255.255.0 172.16.102.1 ip route-static 192.168.3.0 255.255.255.0 172.16.102.1 ip route-static 192.168.4.0 255.255.255.0 172.16.102.1 ip route-static 192.168.5.0 255.255.255.0 172.16.102.1 ip route-static 192.168.6.0 255.255.255.0 172.16.102.1 ip route-static 192.168.7.0 255.255.255.0 172.16.102.1 ip route-static 192.168.8.0 255.255.255.0 172.16.102.1 ip route-static 192.168.9.0 255.255.255.0 172.16.102.1 ip route-static 192.168.10.0 255.255.255.0 172.16.102.1 ip route-static 192.168.11.0 255.255.255.0 172.16.102.1 ip route-static 192.168.12.0 255.255.255.0 172.16.102.1 ip route-static 192.168.14.0 255.255.255.0 172.16.102.1 ip route-static 192.168.18.0 255.255.255.0 172.16.102.1 ip route-static 192.168.19.0 255.255.255.0 172.16.102.1 ip route-static 192.168.20.0 255.255.255.0 172.16.102.1 ip route-static 192.168.27.0 255.255.255.0 172.16.102.1 ip route-static 192.168.60.0 255.255.255.0 172.16.102.1 ip route-static 192.168.63.0 255.255.255.0 172.16.102.1 ## 2号楼网络： ip route-static 192.168.21.0 255.255.255.0 172.16.102.1 ip route-static 192.168.22.0 255.255.255.0 172.16.102.1 ip route-static 192.168.80.0 255.255.248.0 172.16.102.1 ip route-static 192.168.88.0 255.255.255.0 172.16.102.1 ip route-static 192.168.89.0 255.255.255.0 172.16.102.1 ## 各分院网络： ip route-static 192.168.48.0 255.255.248.0 172.16.102.1 description TO_GXYY_48-55 ip route-static 192.168.56.0 255.255.252.0 172.16.102.1 description TO_GXYY_56-59 ip route-static 192.168.70.0 255.255.255.0 172.16.102.1 description MaZhai ip route-static 192.168.75.0 255.255.255.0 172.16.102.1 description TO_KangFu ip route-static 192.168.76.0 255.255.255.0 172.16.102.1 description TO_YuXing ip route-static 192.168.94.0 255.255.255.0 172.16.102.1 description TO_GouZhao ip route-static 192.168.95.0 255.255.255.0 172.16.102.1 description TO_WenHuaGong ip route-static 192.168.97.0 255.255.255.0 172.16.102.1 description TO_WuMian_LaoGanBu ip route-static 192.168.98.0 255.255.255.0 172.16.102.1 description TO_RenDa_JiWei ip route-static 172.17.0.0 255.255.0.0 172.30.200.2 description to FenYuan_XinZheng

## 服务器网段： ip route-static 10.20.0.0 255.255.255.0 172.16.102.1 ip route-static 10.20.1.0 255.255.255.0 172.16.102.1 ip route-static 10.20.2.0 255.255.255.0 172.16.102.1 ip route-static 10.30.0.0 255.255.255.0 172.16.102.1 ip route-static 10.40.0.0 255.255.0.0 172.16.102.1 # ip route-static 10.98.0.0 255.255.0.0 172.16.102.1 # 郑州市卫健委：视频专线 ip route-static 10.167.4.0 255.255.255.0 172.16.102.1 # 郑州市卫健委-卫生专网 ip route-static 10.224.0.0 255.255.0.0 172.16.102.1 ip route-static 10.232.0.0 255.255.0.0 172.16.102.1 # 郑州市卫健委-电子政务外网 ip route-static 59.207.34.0 255.255.255.0 172.16.102.1 ip route-static 59.207.51.0 255.255.255.0 172.16.102.1 ip route-static 59.207.61.0 255.255.255.0 172.16.102.1 # 三层路由互联地址： ip route-static 11.0.2.0 255.255.255.0 172.16.102.1 ip route-static 172.16.100.0 255.255.255.248 172.16.102.1 ip route-static 172.16.101.0 255.255.255.248 172.16.102.1 ip route-static 172.16.103.0 255.255.255.0 172.16.102.1 ip route-static 172.16.104.0 255.255.255.0 172.16.102.1

# 省市医保【省市医保防火墙，华为：USG6507，admin@123)】 ip route-static 59.207.62.136 255.255.255.255 192.168.100.251 description ShiYiBao ip route-static 21.0.255.15 255.255.255.255 192.168.100.251 ip route-static 10.120.0.0 255.255.0.0 192.168.100.251 ip route-static 10.121.0.0 255.255.0.0 192.168.100.251 ip route-static 172.18.192.29 255.255.255.255 192.168.100.251 ip route-static 172.18.192.40 255.255.255.248 192.168.100.251 description shiyibao_jiekou_172.18.192.42 ip route-static 172.18.224.136 255.255.255.248 192.168.100.251 description shengyibao_jk_172.18.224.137 ip route-static 172.120.0.0 255.255.0.0 192.168.100.251 ip route-static 191.191.191.0 255.255.255.0 192.168.100.251 ip route-static 192.1.0.0 255.255.0.0 192.168.100.251

# 省工伤医保（经过信息科防火墙172.16.101.3，进入外网省工伤VPN：192.168.0.65） ip route-static 10.120.6.22 255.255.255.255 192.168.100.253 description gongshangyibao ip route-static 10.120.84.35 255.255.255.255 192.168.100.253 description gongshangyibao1 # 外网交换机（192.168.0.254）上有的路由： ip route-static 10.120.96.54 32 192.168.0.65 description gongshangyibao1 # 高招VPN（康复医院核心交换机192.168.75.251，转接深信服VPN） - 此条路由已经失效，需要配置在5号楼DELL交换机上。 ip route-static 10.255.2.0 255.255.255.0 192.168.75.251 description gaozhaotijian1

# 无效路由 ip route-static 10.100.0.0 255.255.0.0 10.102.3.1 ip route-static 10.102.0.0 255.255.0.0 10.102.3.1 广电（全部弃用） ip route-static 172.16.52.5 255.255.255.255 192.168.100.241 description guangdian ip route-static 172.16.52.6 255.255.255.254 192.168.100.241 description guangdian ip route-static 172.16.52.8 255.255.255.254 192.168.100.241 description guangdian ip route-static 172.30.69.1 255.255.255.255 192.168.100.241 description guangdian ip route-static 172.30.93.76 255.255.255.255 192.168.100.241 description guangdian ip route-static 172.30.95.196 255.255.255.255 192.168.100.241 description guangdian #老农合改走医保专线（全部弃用） ip route-static 172.16.10.0 255.255.255.0 192.168.0.15 ip route-static 172.16.90.0 255.255.255.0 192.168.0.15 # 7.2、柯利尔（建行） ip route-static 15.0.160.35 255.255.255.255 172.16.102.1 description JianHang # 7.4、银联专线（Cisco 路由器：192.168.100.91） ip route-static 11.88.253.240 255.255.255.255 192.168.100.91 ip route-static 11.88.254.0 255.255.255.0 192.168.100.91 ip route-static 11.131.241.71 255.255.255.255 192.168.100.91 # 10.1、无线AC-AP管理 ip route-static 171.19.100.0 255.255.255.0 192.168.15.254 # 7.3、120（院前急救） ip route-static 180.9.0.0 255.255.0.0 172.16.102.1

C）合并后的静态路由表（配置在华为S7703汇聚交换机）【68条】

一、服务器网段：

ip route-static 10.20.0.0 24 172.16.101.1 desc Server_Network

ip route-static 10.20.1.0 24 172.16.101.1 desc Server_Network

ip route-static 10.20.2.0 24 172.16.101.1 desc Server_Network

二、5号楼网络：

ip route-static 192.168.1.0 24 172.16.101.1 desc Building_5#_Network

ip route-static 192.168.2.0 24 172.16.101.1 desc Building_5#_Network

ip route-static 192.168.3.0 24 172.16.101.1 desc Building_5#_Network

ip route-static 192.168.4.0 24 172.16.101.1 desc Building_5#_Network

ip route-static 192.168.5.0 24 172.16.101.1 desc Building_5#_Network

ip route-static 192.168.6.0 24 172.16.101.1 desc Building_5#_Network

ip route-static 192.168.7.0 24 172.16.101.1 desc Building_5#_Network

ip route-static 192.168.8.0 24 172.16.101.1 desc Building_5#_Network

ip route-static 192.168.9.0 24 172.16.101.1 desc Building_5#_Network

ip route-static 192.168.10.0 24 172.16.101.1 desc Building_5#_Network

ip route-static 192.168.11.0 24 172.16.101.1 desc Building_5#_Network

ip route-static 192.168.12.0 24 172.16.101.1 desc Building_5#_Network

ip route-static 192.168.14.0 24 172.16.101.1 desc Building_5#_Network

ip route-static 192.168.18.0 24 172.16.101.1 desc Building_5#_Network

ip route-static 192.168.19.0 24 172.16.101.1 desc Building_5#_Network

# ip route-static 192.168.20.0 24 172.16.101.1 desc Building_5#_Network_BeiYong

ip route-static 192.168.27.0 24 172.16.101.1 desc Building_5#_Network

ip route-static 192.168.60.0 24 172.16.101.1 desc Building_5#_Network

ip route-static 192.168.63.0 24 172.16.101.1 desc Building_5#_Network

三、2号楼网段（内网）

ip route-static 192.168.21.0 24 172.16.101.1 desc Building_2#_Network

ip route-static 192.168.22.0 24 172.16.101.1 desc Building_2#_Network

ip route-static 192.168.80.0 21 172.16.101.1 desc Building_2#_Network

ip route-static 192.168.88.0 24 172.16.101.1 desc Building_2#_Network

ip route-static 192.168.89.0 24 172.16.101.1 desc Building_2#_Network

四、各分院网络

# 4.1、高新院区

ip route-static 192.168.48.0 21 172.16.101.1 desc FenYuan_GaoXinYuanQu

ip route-static 192.168.56.0 22 172.16.101.1 desc FenYuan_GaoXinYuanQu

# 4.2、马寨

ip route-static 192.168.70.0 24 172.16.101.1 desc FenYuan_MaZhai

# 4.3、康复医院

ip route-static 192.168.75.0 24 172.16.101.1 desc FenYuan_KangFu

# 4.4、豫欣医院

ip route-static 192.168.76.0 24 172.16.101.1 desc FenYuan_YuXing

# 4.5、沟赵卫生院

ip route-static 192.168.94.0 24 172.16.101.1 desc FenYuan_GouZhao

# 4.6、文化宫路院区

ip route-static 192.168.95.0 24 172.16.101.1 desc FenYuan_WenHuaGong

# 4.7、老干部活动中心\五棉社区服务站

ip route-static 192.168.97.0 24 172.16.101.1 desc FenYuan_LaoGanBu_WuMian

# 4.8、纪检委\人大

ip route-static 192.168.98.0 24 172.16.101.1 desc FenYuan_JiJianWei_RenDa

# 4.9、新郑市公立医院（启用一个三层接口）

ip route-static 172.17.0.0 255.255.0.0 172.30.200.2 description to FenYuan_XinZheng

五、郑州市卫健委

# 5.1、电子政务外网

ip route-static 59.207.34.0 24 172.16.101.1 desc ZZSWJW_DZZW

ip route-static 59.207.51.0 24 172.16.101.1 desc ZZSWJW_DZZW

ip route-static 59.207.61.0 24 172.16.101.1 desc ZZSWJW_DZZW

# 5.2、卫健委视频专线（防火墙互联地址，在DELL交换机上，VLAN390）

ip route-static 10.167.4.0 24 172.16.101.1 desc ZZSWJW_ShiPingHuiYi

# 5.3、卫生专网

ip route-static 10.224.0.0 16 172.16.101.1 desc ZZSWJW_WeiShengZhuanWang

ip route-static 10.232.0.0 16 172.16.101.1 desc ZZSWJW_WeiShengZhuanWang

六、三层路由互联地址：

ip route-static 11.0.1.0 255.255.255.0 172.16.101.1 description Layer3_HuLian

ip route-static 11.0.2.0 255.255.255.0 172.16.101.1 description Layer3_HuLian

ip route-static 172.16.100.0 255.255.255.0 172.16.101.1 description Layer3_HuLian

ip route-static 172.16.103.0 255.255.255.0 172.16.101.1 description Layer3_HuLian

七、其他专线：

# 7.1、郑大一附院，远程医疗

ip route-static 10.100.0.0 16 172.16.101.1 desc ZhuanXian_ZDYFY

ip route-static 10.102.0.0 16 172.16.101.1 desc ZhuanXian_ZDYFY

# 7.2、柯利尔（建行）

ip route-static 15.0.160.35 32 172.16.101.1 desc ZhuanXian_Clear_JianHang

# 7.3、120（院前急救）

ip route-static 180.9.0.0 16 172.16.101.1 desc ZhuanXian_120_YuanQianJiJiu

# 7.4、银联专线（Cisco 路由器：192.168.100.91）

ip route-static 11.88.253.240 255.255.255.255 192.168.100.91 desc ZhuanXian_YinLian

ip route-static 11.88.254.0 255.255.255.0 192.168.100.91 desc ZhuanXian_YinLian

ip route-static 11.131.241.71 255.255.255.255 192.168.100.91 desc ZhuanXian_YinLian

八、省市医保（8条静态路由在使用）

【省市医保防火墙192.168.100.251，华为：USG6507，admin@123)】

# 省医保

ip route-static 10.85.96.36 255.255.255.255 192.168.100.251 desc YB_ShengYiBao

ip route-static 192.1.0.0 255.255.0.0 192.168.100.251 desc YB_ShengYiBao

# 省市医保公用

ip route-static 10.120.0.0 255.255.0.0 192.168.100.251 desc YB_Sheng_Shi_YiBao

# 市医保

ip route-static 10.121.0.0 255.255.0.0 192.168.100.251 desc YB_ShiYiBao

ip route-static 59.207.62.136 255.255.255.255 192.168.100.251 desc YB_ShiYiBao

ip route-static 172.120.0.0 255.255.0.0 192.168.100.251 desc YB_ShiYiBao

ip route-static 191.191.191.0 255.255.255.0 192.168.100.251 desc YB_ShiYiBao

# 卫生局大数据：

ip route-static 21.0.255.15 255.255.255.255 192.168.100.251 desc YB_WeiShengJu_DaShuJu

# 未确定（可能不用了）：【确定没有用】

ip route-static 172.18.192.29 255.255.255.255 192.168.100.251 desc YB

ip route-static 172.18.192.40 255.255.255.248 192.168.100.251 desc YB_shiyibao_jiekou_172.18.192.42

ip route-static 172.18.224.136 255.255.255.248 192.168.100.251 desc YB_shengyibao_jk_172.18.224.137

九、省工伤医保【经过信息科防火墙路由，华为USG安全网关，172.16.101.3】

ip route-static 10.120.84.35 255.255.255.255 172.16.101.3 desc YB_ShengGongShang

ip route-static 10.120.96.54 255.255.255.255 172.16.101.3 desc YB_ShengGongShang

十、其他路由

10.1、无线AC（172.16.100.0 AP管理地址段，int vlan119: 192.168.15.1）

ip route-static 171.19.100.0 255.255.255.0 192.168.15.254 desc WiFi_AP_GuanLi

10.1、高招VPN（康复医院核心交换机192.168.75.251，转接深信服VPN）

此条路由已经失效，应改配置在5号楼DELL交换机上。

# ip route-static 10.255.2.0 255.255.255.0 192.168.75.251 desc gaozhaotijian1

D）最终静态路由表【64条】

ip route-static 0.0.0.0 0 172.16.101.3 desc TO_Huawei_USG

ip route-static 10.20.0.0 255.255.255.0 172.16.101.1 description Server_Network ip route-static 10.20.1.0 255.255.255.0 172.16.101.1 description Server_Network ip route-static 10.20.2.0 255.255.255.0 172.16.101.1 description Server_Network ip route-static 10.85.96.36 255.255.255.255 192.168.100.251 description YB_ShengYiBao ip route-static 10.100.0.0 255.255.0.0 172.16.101.1 description ZhuanXian_ZDYFY ip route-static 10.102.0.0 255.255.0.0 172.16.101.1 description ZhuanXian_ZDYFY ip route-static 10.120.0.0 255.255.0.0 192.168.100.251 description YB_Sheng_Shi_YiBao ip route-static 10.120.84.35 255.255.255.255 172.16.101.3 description YB_ShengGongShang ip route-static 10.120.96.54 255.255.255.255 172.16.101.3 description YB_ShengGongShang ip route-static 10.121.0.0 255.255.0.0 192.168.100.251 description YB_ShiYiBao ip route-static 10.167.4.0 255.255.255.0 172.16.101.1 description ZZSWJW_ShiPingHuiYi ip route-static 10.224.0.0 255.255.0.0 172.16.101.1 description ZZSWJW_WeiShengZhuanWang ip route-static 10.232.0.0 255.255.0.0 172.16.101.1 description ZZSWJW_WeiShengZhuanWang+ ip route-static 11.0.1.0 255.255.255.0 172.16.101.1 description Layer3_HuLian ip route-static 11.0.2.0 255.255.255.0 172.16.101.1 description Layer3_HuLian ip route-static 11.88.253.240 255.255.255.255 192.168.100.91 description ZhuanXian_YinLian ip route-static 11.88.254.0 255.255.255.0 192.168.100.91 description ZhuanXian_YinLian ip route-static 11.131.241.71 255.255.255.255 192.168.100.91 description ZhuanXian_YinLian ip route-static 15.0.160.35 255.255.255.255 172.16.101.1 description ZhuanXian_Clear_JianHang ip route-static 21.0.255.15 255.255.255.255 192.168.100.251 description YB_WeiShengJu_DaShuJu ip route-static 59.207.34.0 255.255.255.0 172.16.101.1 description ZZSWJW_DZZW ip route-static 59.207.51.0 255.255.255.0 172.16.101.1 description ZZSWJW_DZZW ip route-static 59.207.61.0 255.255.255.0 172.16.101.1 description ZZSWJW_DZZW ip route-static 59.207.62.136 255.255.255.255 192.168.100.251 description YB_ShiYiBao ip route-static 171.19.100.0 255.255.255.0 192.168.15.254 description WiFi_AP_GuanLi ip route-static 172.16.100.0 255.255.255.0 172.16.101.1 description Layer3_HuLian ip route-static 172.16.103.0 255.255.255.0 172.16.101.1 description Layer3_HuLian ip route-static 172.17.0.0 255.255.0.0 172.30.200.2 description to FenYuan_XinZheng ip route-static 172.120.0.0 255.255.0.0 192.168.100.251 description YB_ShiYiBao ip route-static 180.9.0.0 255.255.0.0 172.16.101.1 description ZhuanXian_120_YuanQianJiJiu ip route-static 191.191.191.0 255.255.255.0 192.168.100.251 description YB_ShiYiBao ip route-static 192.1.0.0 255.255.0.0 192.168.100.251 description YB_ShengYiBao ip route-static 192.168.1.0 255.255.255.0 172.16.101.1 description Building_5#_Network ip route-static 192.168.2.0 255.255.255.0 172.16.101.1 description Building_5#_Network ip route-static 192.168.3.0 255.255.255.0 172.16.101.1 description Building_5#_Network ip route-static 192.168.4.0 255.255.255.0 172.16.101.1 description Building_5#_Network ip route-static 192.168.5.0 255.255.255.0 172.16.101.1 description Building_5#_Network ip route-static 192.168.6.0 255.255.255.0 172.16.101.1 description Building_5#_Network ip route-static 192.168.7.0 255.255.255.0 172.16.101.1 description Building_5#_Network ip route-static 192.168.8.0 255.255.255.0 172.16.101.1 description Building_5#_Network ip route-static 192.168.9.0 255.255.255.0 172.16.101.1 description Building_5#_Network ip route-static 192.168.10.0 255.255.255.0 172.16.101.1 description Building_5#_Network ip route-static 192.168.11.0 255.255.255.0 172.16.101.1 description Building_5#_Network ip route-static 192.168.12.0 255.255.255.0 172.16.101.1 description Building_5#_Network ip route-static 192.168.14.0 255.255.255.0 172.16.101.1 description Building_5#_Network ip route-static 192.168.18.0 255.255.255.0 172.16.101.1 description Building_5#_Network ip route-static 192.168.19.0 255.255.255.0 172.16.101.1 description Building_5#_Network ip route-static 192.168.21.0 255.255.255.0 172.16.101.1 description Building_2#_Network ip route-static 192.168.22.0 255.255.255.0 172.16.101.1 description Building_2#_Network ip route-static 192.168.27.0 255.255.255.0 172.16.101.1 description Building_5#_Network ip route-static 192.168.48.0 255.255.248.0 172.16.101.1 description FenYuan_GaoXinYuanQu ip route-static 192.168.56.0 255.255.252.0 172.16.101.1 description FenYuan_GaoXinYuanQu ip route-static 192.168.60.0 255.255.255.0 172.16.101.1 description Building_5#_Network ip route-static 192.168.63.0 255.255.255.0 172.16.101.1 description Building_5#_Network ip route-static 192.168.70.0 255.255.255.0 172.16.101.1 description FenYuan_MaZhai ip route-static 192.168.75.0 255.255.255.0 172.16.101.1 description FenYuan_KangFu ip route-static 192.168.76.0 255.255.255.0 172.16.101.1 description FenYuan_YuXing ip route-static 192.168.80.0 255.255.248.0 172.16.101.1 description Building_2#_Network ip route-static 192.168.88.0 255.255.255.0 172.16.101.1 description Building_2#_Network ip route-static 192.168.89.0 255.255.255.0 172.16.101.1 description Building_2#_Network ip route-static 192.168.94.0 255.255.255.0 172.16.101.1 description FenYuan_GouZhao ip route-static 192.168.95.0 255.255.255.0 172.16.101.1 description FenYuan_WenHuaGong ip route-static 192.168.97.0 255.255.255.0 172.16.101.1 description FenYuan_LaoGanBu_WuMian ip route-static 192.168.98.0 255.255.255.0 172.16.101.1 description FenYuan_JiJianWei_RenDa

- 连接外网交换机

- 配置1号楼外网汇聚华为S5720-32P-EI（192.168.101.241）：

interface GigabitEthernet0/0/23

description TO_Huawei_S7703_Core

port link-type trunk

undo port trunk allow-pass vlan 1

port trunk allow-pass vlan 100 to 101 104

- 配置1号楼内网汇聚：华为S7703

interface GigabitEthernet2/0/23

description TO_Huawei_S5720(wai)_192.168.101.241_p23

port link-type trunk

undo port trunk allow-pass vlan 1

port trunk allow-pass vlan 100 to 101 104

- 关于切换192.168.100.254网关部分

- 华为新交换机S7703：改路由（医保、银联）

undo interface Vlan-interface1000

改路由：192.168.100.251、192.168.100.91

undo ip route-static 10.85.96.36 255.255.255.255 192.168.100.251

undo ip route-static 10.120.0.0 255.255.0.0 192.168.100.251

undo ip route-static 10.121.0.0 255.255.0.0 192.168.100.251

undo ip route-static 21.0.255.15 255.255.255.255 192.168.100.251

undo ip route-static 59.207.62.136 255.255.255.255 192.168.100.251

undo ip route-static 172.120.0.0 255.255.0.0 192.168.100.251

undo ip route-static 191.191.191.0 255.255.255.0 192.168.100.251

undo ip route-static 192.1.0.0 255.255.0.0 192.168.100.251

ip route-static 10.85.96.36 255.255.255.255 172.16.101.1 desc YB_ShengYiBao

ip route-static 10.120.0.0 255.255.0.0 172.16.101.1 desc YB_Sheng_Shi_YiBao

ip route-static 10.121.0.0 255.255.0.0 172.16.101.1 desc YB_ShiYiBao

ip route-static 21.0.255.15 255.255.255.255 172.16.101.1 desc YB_WeiShengJu_DaShuJu

ip route-static 59.207.62.136 255.255.255.255 172.16.101.1 desc YB_ShiYiBao

ip route-static 172.120.0.0 255.255.0.0 172.16.101.1 desc YB_ShiYiBao

ip route-static 191.191.191.0 255.255.255.0 172.16.101.1 desc YB_ShiYiBao

ip route-static 192.1.0.0 255.255.0.0 172.16.101.1 desc YB_ShengYiBao

undo ip route-static 11.88.253.240 255.255.255.255 192.168.100.91

undo ip route-static 11.88.254.0 255.255.255.0 192.168.100.91

undo ip route-static 11.131.241.71 255.255.255.255 192.168.100.91

ip route-static 11.88.253.240 255.255.255.255 172.16.101.1 desc ZhuanXian_YinLian

ip route-static 11.88.254.0 255.255.255.0 172.16.101.1 desc ZhuanXian_YinLian

ip route-static 11.131.241.71 255.255.255.255 172.16.101.1 desc ZhuanXian_YinLian

ip route-static 192.168.100.0 255.255.255.0 172.16.101.1 desc Server_Network

- 修改2号楼华为核心交换机S12708：改路由（医保、银联）

interface Vlan-interface1000

ip address 192.168.100.254 255.255.255.0

改路由：192.168.100.251、192.168.100.91

ip route-st 10.85.96.36 255.255.255.255 192.168.100.251 desc YB_ShengYiBao

ip route-st 10.120.0.0 255.255.0.0 192.168.100.251 desc YB_Sheng_Shi_YiBao

ip route-st 10.121.0.0 255.255.0.0 192.168.100.251 desc YB_ShiYiBao

ip route-st 21.0.255.15 255.255.255.255 192.168.100.251 desc YB_WeiShengJu_DaShuJu

ip route-st 59.207.62.136 255.255.255.255 192.168.100.251 desc YB_ShiYiBao

ip route-st 172.120.0.0 255.255.0.0 192.168.100.251 desc YB_ShiYiBao

ip route-st 191.191.191.0 255.255.255.0 192.168.100.251 desc YB_ShiYiBao

ip route-st 192.1.0.0 255.255.0.0 192.168.100.251 desc YB_ShengYiBao

ip route-st 11.88.253.240 255.255.255.255 192.168.100.91 desc ZhuanXian_YinLian

ip route-st 11.88.254.0 255.255.255.0 192.168.100.91 desc ZhuanXian_YinLian

ip route-st 11.131.241.71 255.255.255.255 192.168.100.91 desc ZhuanXian_YinLian

- 配置信息科内外网防火墙（华为防火墙）

1）防火墙路由表：

路由到内网网段：？？？

ip route-static 10.20.0.0 255.255.255.0 172.16.101.2 desc Server_Network ip route-static 10.20.1.0 255.255.255.0 172.16.101.2 desc Server_Network ip route-static 10.20.2.0 255.255.255.0 172.16.101.2 desc Server_Network ip route-static 10.85.96.36 255.255.255.255 172.16.101.2 desc YB_ShengYiBao ip route-static 10.100.0.0 255.255.0.0 172.16.101.2 desc ZhuanXian_ZDYFY ip route-static 10.102.0.0 255.255.0.0 172.16.101.2 desc ZhuanXian_ZDYFY ip route-static 10.120.0.0 255.255.0.0 172.16.101.2 desc YB_Sheng_Shi_YiBao ip route-static 10.120.84.35 255.255.255.255 10.10.12.254 desc YB_ShengGongShang ip route-static 10.120.96.54 255.255.255.255 10.10.12.254 desc YB_ShengGongShang ip route-static 10.121.0.0 255.255.0.0 172.16.101.2 desc YB_ShiYiBao ip route-static 10.167.4.0 255.255.255.0 172.16.101.2 desc ZZSWJW_ShiPingHuiYi ip route-static 10.224.0.0 255.255.0.0 172.16.101.2 desc ZZSWJW_WeiShengZhuanWang ip route-static 10.232.0.0 255.255.0.0 172.16.101.2 desc ZZSWJW_WeiShengZhuanWang+ ip route-static 11.0.1.0 255.255.255.0 172.16.101.2 desc Layer3_HuLian ip route-static 11.0.2.0 255.255.255.0 172.16.101.2 desc Layer3_HuLian ip route-static 11.88.253.240 255.255.255.255 172.16.101.2 desc ZhuanXian_YinLian ip route-static 11.88.254.0 255.255.255.0 172.16.101.2 desc ZhuanXian_YinLian ip route-static 11.131.241.71 255.255.255.255 172.16.101.2 desc ZhuanXian_YinLian ip route-static 15.0.160.35 255.255.255.255 172.16.101.2 desc ZhuanXian_Clear_JianHang ip route-static 21.0.255.15 255.255.255.255 172.16.101.2 desc YB_WeiShengJu_DaShuJu ip route-static 59.207.34.0 255.255.255.0 172.16.101.2 desc ZZSWJW_DZZW ip route-static 59.207.51.0 255.255.255.0 172.16.101.2 desc ZZSWJW_DZZW ip route-static 59.207.61.0 255.255.255.0 172.16.101.2 desc ZZSWJW_DZZW ip route-static 59.207.62.136 255.255.255.255 172.16.101.2 desc YB_ShiYiBao ip route-static 171.19.100.0 255.255.255.0 172.16.101.2 desc WiFi_AP_GuanLi ip route-static 172.16.100.0 255.255.255.0 172.16.101.2 desc Layer3_HuLian ip route-static 172.16.103.0 255.255.255.0 172.16.101.2 desc Layer3_HuLian ip route-static 172.17.0.0 255.255.0.0 172.30.200.2 desc to FenYuan_XinZheng ip route-static 172.120.0.0 255.255.0.0 172.16.101.2 desc YB_ShiYiBao ip route-static 180.9.0.0 255.255.0.0 172.16.101.2 desc ZhuanXian_120_YuanQianJiJiu ip route-static 191.191.191.0 255.255.255.0 172.16.101.2 desc YB_ShiYiBao ip route-static 192.1.0.0 255.255.0.0 172.16.101.2 desc YB_ShengYiBao ip route-static 192.168.0.0 255.255.255.0 10.10.12.254 ip route-static 192.168.0.0 255.255.0.0 172.16.101.2 desc HIS_LAN

#缺省路由： ip route-static 0.0.0.0 0 10.10.12.254 description Default_to_192.168.0.254

2）修改外网核心交换机（192.168.0.254）路由表：

原指向内网的静态路由：

ip route-static 10.20.0.0 24 192.168.100.254

ip route-static 10.20.1.0 24 192.168.100.254

ip route-static 10.40.124.0 24 192.168.100.254

ip route-static 172.16.101.0 29 192.168.100.254 description DELL

ip route-static 172.16.103.0 24 192.168.100.254

ip route-static 192.168.0.0 18 192.168.100.254 description 0-63

ip route-static 192.168.64.0 19 192.168.100.254 description 64-95

ip route-static 192.168.96.0 22 192.168.100.254 description 96-99

ip route-static 192.168.100.91 32 192.168.100.254

ip route-static 192.168.100.251 32 192.168.100.254

删除原指向内网的静态路由：

undo ip route-static 10.20.0.0 24 192.168.100.254

undo ip route-static 10.20.1.0 24 192.168.100.254

undo ip route-static 10.40.124.0 24 192.168.100.254

undo ip route-static 172.16.101.0 29 192.168.100.254

undo ip route-static 172.16.103.0 24 192.168.100.254

undo ip route-static 192.168.0.0 18 192.168.100.254

undo ip route-static 192.168.64.0 19 192.168.100.254

undo ip route-static 192.168.96.0 22 192.168.100.254

undo ip route-static 192.168.100.91 32 192.168.100.254

undo ip route-static 192.168.100.251 32 192.168.100.254

添加新的指向内网的静态路由（指向华为USG 6309防火墙）：

ip route-static 10.20.0.0 24 10.10.12.250

ip route-static 10.20.1.0 24 10.10.12.250

ip route-static 10.40.124.0 24 10.10.12.250

ip route-static 172.16.101.0 29 10.10.12.250 description DELL

ip route-static 172.16.103.0 24 10.10.12.250

ip route-static 192.168.0.0 18 10.10.12.250 description 0-63

ip route-static 192.168.64.0 19 10.10.12.250 description 64-95

ip route-static 192.168.96.0 22 10.10.12.250 description 96-99

ip route-static 192.168.100.91 32 10.10.12.250

ip route-static 192.168.100.251 32 10.10.12.250

3）修改1号楼汇聚交换机（172.16.101.2）路由表：

ip route-static 10.10.0.0 16 172.16.101.3

4）修改2号楼核心交换机（172.16.101.1）路由表：

ip route-static 10.10.0.0 16 172.16.101.2

- 配置2号楼核心交换机S12708（10.20.0.254）交换机

- 配置到1号楼华为 S7703交换机的互联接口（仅修改VLAN Trunk）

| 现有S12708配置 | 修改后的S12708配置 |
| --- | --- |
| interface Eth-Trunk8 description to-1haolou-dell-s4048t port link-type trunk undo port trunk allow-pass vlan 1 port trunk allow-pass vlan 62 119 to 120 300 to 301 1000 2013 2999 3001 3004 traffic-filter inbound acl 3003 traffic-filter outbound acl 3003 mode lacp load-balance dst-mac port-mirroring to observe-port 3 inbound port-mirroring to observe-port 3 outbound | interface Eth-Trunk8 description to-1haolou-dell-s4048t port link-type trunk undo port trunk allow-pass vlan 1 port trunk allow-pass vlan 62 119 to 120 300 2013 3001 traffic-filter inbound acl 3003 traffic-filter outbound acl 3003 mode lacp load-balance dst-mac port-mirroring to observe-port 3 inbound port-mirroring to observe-port 3 outbound |
| interface XGigabitEthernet7/0/2 description to-1haolou-dell-s4048t#1_1/52/1 eth-trunk 8 interface XGigabitEthernet7/0/3 description to-1haolou-dell-s4048t#2_2/52/1 eth-trunk 8 | interface XGigabitEthernet7/0/2 description to-1haolou-dell-s4048t#1_1/52/1 eth-trunk 8 interface XGigabitEthernet7/0/3 description to-1haolou-dell-s4048t#2_2/52/1 eth-trunk 8 |

配置到省工伤的路由（经过华为防火墙）：

ip route-static 10.120.84.35 255.255.255.255 172.16.101.3 desc YB_ShengGongShang

ip route-static 10.120.96.54 255.255.255.255 172.16.101.3 desc YB_ShengGongShang

- 改造与变更流程

- 保障方案

- 指挥组

连军胜、李晓莉

- 本次改造影响的楼层范围

门诊楼

急诊楼

门诊2楼远程PACS诊断室

1号楼C座1楼

- 发布网络改造维护通知

- 技术保障与分工

| 序号 | 姓名 | 手机号码 | 部门 | 分工 |
| --- | --- | --- | --- | --- |
| 1 | 李常福 | 13633825095 | 信息科 | 网络数据配置 |
| 2 | 王凌宇 | 15670612817 | 信息科 | 网络数据配置 |
| 3 | 王寅 | 13598847384 | 信息科 | 设备搬迁、线标 |
| 4 | 杨匀正 | 18638558221 | 信息科 | 安全设备迁移 |
| 5 | 熊文伟 | 18503713334 | 第三方运维 | 现场网络调试 |
| 6 | 张飞 | 15890904714 | 第三方运维 | 设备搬迁、线标 |
| 7 |  |  | 网络工程师 | 设备供货商 |

- 切换计划

第一阶段迁移计划：在2021年7月9日前：

- 完成192.168.100.0/24段迁移

- 完成华为S7703交换机上架与配置

- 完成需要迁移网线、光纤跳线标签打印

第二阶段迁移计划：

| 日期 | 时间 | 变更操作内容 |
| --- | --- | --- |
| 一、准备工作 |  |  |
| 7月23日 | 12:00前完成 | 1、准备设备上架机柜螺丝，标签数据； 2、准备充足的电源线、光纤跳线，扎带； 3、推车，打光笔，寻线仪，5米网线15根，RJ45头，网钳，笔记本电脑2台，串口线。 |
| 7月23日 | 20:00-21:00 | 做好相关的网络设备配置备份 |
| 二、调整配置 |  |  |
| 7月23日 | 21:00-21:20 | 配置华为交换机，调整光纤与网线连接 |
| 7月23日 | 21:20-21:40 | 配置2号楼核心交换机 |
| 7月23日 | 21:40-22:00 | 验证配置，测试各系统连通性 |
| 7月23日 | 22:00 | 完成改造，后续跟进理线与标签打印。 |

- 切换后，测试网络连通性：

| 服务器网段 | 10.20.0.254 10.20.1.254 10.20.2.254 |
| --- | --- |
| 5号楼网络 | 192.168.1.254 192.168.2.254 192.168.63.254 |
| 2号楼网段 | 192.168.21.254 192.168.89.254 |
| 各分院网络 | 192.168.51.254 192.168.55.254 192.168.70.254 192.168.75.254 192.168.76.254 192.168.94.254 192.168.95.254 192.168.97.254 192.168.98.254 |
| 新郑分院 | 172.17.0.0 |
| 郑州市卫健委 | 59.207.34.0 59.207.51.0 59.207.61.0 10.167.4.0 10.224.0.0 10.232.0.0 |
| 其他专线：郑大一附院，远程医疗 | 10.100.0.0 10.102.0.0 |
| 其他专线：柯利尔（建行） | 15.0.160.35 |
| 其他专线：120（院前急救） | 180.9.0.0 |
| 其他专线：银联专线 | 192.168.100.91 11.88.253.240 11.131.241.71 |
| 省市医保 | 10.85.96.36 21.0.255.15 59.207.62.136 172.18.192.29 |
| 省工伤医保 | 10.120.6.22 10.120.84.35 10.120.96.54 |

- 应急措施

启动迁移后，如果出现问题，各方应在30分钟内排查解决。如果无法在短时间内解决，恢复以前线路配置，线路还原，网络数据配置还原，分析原因，切换另行通知。

实施人员签字：

监督人员签字：

科长签字：

确认日期：